|  |  | OPC UA Companion Specification |
| --- | --- | --- |
| OPC 40082-4 |  |  |
| OPC UA interfaces for plastics and rubber machinery - Peripheral devices Part 4: Dosing Systems Release 1.0.1 2026-10-01 OPC 40082-4 (Release 1.0.1) is identical with VDMA 40082-4:2026-10 and EUROMAP 82.4 (Release 01 October 2026) |  |  |

Contents

Page

Forewords	xi

1	Scope	1

2	Normative references	1

3	Terms, definitions and conventions	2

3.1	Overview	2

3.2	Conventions used in this document	2

3.3	Abbreviations	2

4	General information to OPC UA interfaces for plastics and rubber machinery and OPC UA	2

5	Use cases	3

6	OPC 40082-4 Information Model overview	3

7	DosingSystemType	4

7.1	DosingSystemType definition	4

7.1.1	Machine State and Operation Mode	6

7.2	DeviceMappingNumber	7

7.3	MachineConfiguration	7

7.4	DisplayLanguage	7

7.5	IsContinuous	7

7.6	Operation	7

7.7	Feeder	7

8	OperationType	8

8.1	OperationType definition	8

8.2	IsActive	9

8.3	SetActive	9

8.4	IdentifyDevice	9

8.5	EnableDevice	10

8.6	DisableDevice	10

8.7	RemoteControlOpcUa	10

8.8	EnableDosingOpcUa	10

8.9	StartDosingShot	10

8.10	StartDosingContinuous	11

8.11	StopDosing	11

8.12	StopDosingAfterCycle	11

8.13	Components	11

8.14	Throughput	12

8.15	ShotWeight	12

8.16	DosingDuration	12

8.17	Mixing	12

8.18	BatchWeight	12

8.19	HighestActiveAlarmSeverity	12

8.20	ActiveErrors	12

8.21	ResetAllErrors	12

8.22	ResetErrorById	12

8.23	FirstFilling	13

8.24	StartPurge	13

8.25	PurgeDuration	13

8.26	ElectricalEnergy	13

8.27	Maintenance	14

8.28	ProductionDatasetManagement	14

9	MixingType	14

9.1	MixingType definition	14

9.2	MixingMode	14

9.3	MixingInterval	14

9.4	MixingDuration	14

9.5	IdleMixing	15

10	ComponentsType	15

10.1	ComponentsType definition	15

11	ComponentType	16

11.1	ComponentType definition	16

11.2	IsPresent	17

11.3	SetActive	17

11.4	IsActive	17

11.5	State	17

11.6	FeedrateScalingFactor	18

11.7	Throughput	18

11.8	DosingMode	18

11.9	ElectricalEnergy	18

11.10	Material	18

11.11	Weight	18

11.12	BulkDensity	19

11.13	IdentifyComponent	19

11.14	StartCleaning	19

11.15	AbortCleaning	19

11.16	Levels	20

11.17	MaterialConsumption	20

11.18	StartCalibration	20

11.19	AbortCalibration	20

11.20	ContinuousComponentType	20

11.20.1	ContinuousComponentType definition	20

11.20.2	Percentage	21

11.20.3	WeighingProcess	21

11.20.4	DrivePercentage	21

11.20.5	StartPriming	21

11.20.6	StopPriming	21

11.21	BatchComponentType	22

11.21.1	BatchComponentType definition	22

11.21.2	Percentage	22

12	LevelsType	23

12.1	LevelsType definition	23

12.2	Level	23

12.3	Filling	23

12.4	StartFilling	23

12.5	AbortFilling	23

13	DosingMaterialType	25

13.1	DosingMaterialType definition	25

13.2	MaterialClass	25

14	MaterialConsumptionType	26

14.1	MaterialConsumptionType definition	26

14.2	TotalConsumption	26

14.3	Scrap	26

14.4	Reset	26

15	WeighingProcessType	26

15.1	WeighingProcessType definition	26

15.2	TestWeight	27

15.3	WeighingTime	27

16	DosingMaintenanceType	27

16.1	DosingMaintenanceType definition	27

17	Alarmmanagement	27

18	Profiles and Conformance Units	27

18.1	Conformance Units	27

18.2	Profiles	28

18.2.1	Profile list	28

18.2.2	Server Facets	28

19	Namespaces	29

19.1	Namespace Metadata	29

19.2	Handling of OPC UA Namespaces	29

Annex A (normative)  OPC 40082-4 Namespace and mappings	31

Figures

Figure 1 – Dosing System example	3

Figure 2 – Overview DosingSystemType	4

Figure 3 – MachineryItemState Overview	6

Figure 4 – MachineryOperationMode Overview	6

Figure 5 – OperationType Overview	8

Figure 6 – ComponentType Overview	16

Tables

Table 1 – DosingSystemType Definition	5

Table 2 – DosingSystemType Additional Subcomponents	5

Table 3 – DosingSystemType Additional References	5

Table 4 – OperationType Definition	9

Table 5 – IdentifyDevice Method AddressSpace Definition	9

Table 6 – EnableDevice Method AddressSpace Definition	10

Table 7 – DisableDevice Method AddressSpace Definition	10

Table 8 – StartDosingShot Method AddressSpace Definition	11

Table 9 – StartDosingContinuous Method AddressSpace Definition	11

Table 10 – StopDosing Method AddressSpace Definition	11

Table 11 – StopDosingAfterCycle Method AddressSpace Definition	11

Table 12 – ResetErrorById Method Arguments	13

Table 13 – ResetErrorById Method AddressSpace Definition	13

Table 14 – FirstFilling Method AddressSpace Definition	13

Table 15 – StartPurge Method AddressSpace Definition	13

Table 16 – MixingType definition	14

Table 17 – Values for MixingMode	14

Table 18 – ComponentsType Definition	15

Table 19 – ComponentType Definition	17

Table 20 – ComponentStateEnumeration Definition	18

Table 21 – DosingModeEnumeration Definition	18

Table 22 – IdentifyComponent Method AddressSpace Definition	19

Table 23 – StartCleaning Method AddressSpace Definition	19

Table 24 – AbortCleaning Method AddressSpace Definition	19

Table 25 – StartCalibration Method AddressSpace Definition	20

Table 26 – AbortCalibration Method AddressSpace Definition	20

Table 27 – ContinuousComponentType Definition	21

Table 28 – StartPrimingMethod AddressSpace Definition	21

Table 29 – StopPrimingMethod AddressSpace Definition	21

Table 30 – BatchComponentType Definition	22

Table 31 – LevelsType Definition	23

Table 32 – StartFilling Method AddressSpace Definition	23

Table 33 – AbortFilling Method AddressSpace Definition	23

Table 34 – DosingMaterialType definition	25

Table 35 – Values for MaterialClass	25

Table 36: Calculation Example based on Regrind MaterialClass	25

Table 37 – MaterialConsumptionType definition	26

Table 38 – Reset Method AddressSpace Definition	26

Table 39 – WeighingProcessType Definition	27

Table 40 – DosingMaintenanceType Definition	27

Table 41 – Conformance Units for OPC 40082-4	28

Table 42 – Profile URIs for OPC 40082-4	28

Table 43 – OPC 40082-4 Basic Server Profile	28

Table 44 – OPC 40082-4 Alarms Server Facet	28

Table 45 – NamespaceMetadata Object for this Document	29

Table 46 – Namespaces used in a OPC 40082-4 Server	30

Table 47 – Namespaces used in this document	30

OPC Foundation / EUROMAP

____________

AGREEMENT OF USE

COPYRIGHT RESTRICTIONS

- This document is provided "as is" by the OPC Foundation and EUROMAP.

- Right of use for this specification is restricted to this specification and does not grant rights of use for referred documents.

- Right of use for this specification will be granted without cost.

- This document may be distributed through computer systems, printed or copied as long as the content remains unchanged and the document is not modified.

- OPC Foundation and EUROMAP do not guarantee usability for any purpose and shall not be made liable for any case using the content of this document.

- The user of the document agrees to indemnify OPC Foundation and EUROMAP and their officers, directors and agents harmless from all demands, claims, actions, losses, damages (including damages from personal injuries), costs and expenses (including attorneys' fees) which are in any way related to activities associated with its use of content from this specification.

- The document shall not be used in conjunction with company advertising, shall not be sold or licensed to any party.

- The intellectual property and copyright is solely owned by the OPC Foundation and EUROMAP.

PATENTS

The attention of adopters is directed to the possibility that compliance with or adoption of OPC or EUROMAP specifications may require use of an invention covered by patent rights. OPC Foundation or EUROMAP shall not be responsible for identifying patents for which a license may be required by any OPC or EUROMAP specification, or for conducting legal inquiries into the legal validity or scope of those patents that are brought to its attention. OPC or EUROMAP specifications are prospective and advisory only. Prospective users are responsible for protecting themselves against liability for infringement of patents.

WARRANTY AND LIABILITY DISCLAIMERS

WHILE THIS PUBLICATION IS BELIEVED TO BE ACCURATE, IT IS PROVIDED "AS IS" AND MAY CONTAIN ERRORS OR MISPRINTS. THE OPC FOUDATION NOR EUROMAP MAKES NO WARRANTY OF ANY KIND, EXPRESSED OR IMPLIED, WITH REGARD TO THIS PUBLICATION, INCLUDING BUT NOT LIMITED TO ANY WARRANTY OF TITLE OR OWNERSHIP, IMPLIED WARRANTY OF MERCHANTABILITY OR WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE OR USE. IN NO EVENT SHALL THE OPC FOUNDATION NOR EUROMAP BE LIABLE FOR ERRORS CONTAINED HEREIN OR FOR DIRECT, INDIRECT, INCIDENTAL, SPECIAL, CONSEQUENTIAL, RELIANCE OR COVER DAMAGES, INCLUDING LOSS OF PROFITS, REVENUE, DATA OR USE, INCURRED BY ANY USER OR ANY THIRD PARTY IN CONNECTION WITH THE FURNISHING, PERFORMANCE, OR USE OF THIS MATERIAL, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

The entire risk as to the quality and performance of software developed using this specification is borne by you.

RESTRICTED RIGHTS LEGEND

This Specification is provided with Restricted Rights. Use, duplication or disclosure by the U.S. government is subject to restrictions as set forth in (a) this Agreement pursuant to DFARs 227.7202-3(a); (b) subparagraph (c)(1)(i) of the Rights in Technical Data and Computer Software clause at DFARs 252.227-7013; or (c) the Commercial Computer Software Restricted Rights clause at FAR 52.227-19 subdivision (c)(1) and (2), as applicable. Contractor / manufacturer are the OPC Foundation, 16101 N. 82nd Street, Suite 3B, Scottsdale, AZ, 85260-1830.

COMPLIANCE

The combination of EUROMAP and OPC Foundation shall at all times be the sole entities that may authorize developers, suppliers and sellers of hardware and software to use certification marks, trademarks or other special designations to indicate compliance with these materials as specified within this document. Products developed using this specification may claim compliance or conformance with this specification if and only if the software satisfactorily meets the certification requirements set by EUROMAP or the OPC Foundation. Products that do not meet these requirements may claim only that the product was based on this specification and must not claim compliance or conformance with this specification.

TRADEMARKS

Most computer and software brand names have trademarks or registered trademarks. The individual trademarks have not been listed here.

GENERAL PROVISIONS

Should any provision of this Agreement be held to be void, invalid, unenforceable or illegal by a court, the validity and enforceability of the other provisions shall not be affected thereby.

This Agreement shall be governed by and construed under the laws of Germany.

This Agreement embodies the entire understanding between the parties with respect to, and supersedes any prior understanding or agreement (oral or written) relating to, this specification.

ISSUE REPORTING

If an error or problem is found in this specification, the UaNodeSet, or any associated supplementary files, it should be reported as an issue.

The reporting process can be found here: https://opcfoundation.org/resources/issue-tracking/

The Link to the issue tracking project for this document is here:

https://mantis.opcfoundation.org/set_project.php?project_id=249&make_default=no

Forewords

OPC UA is a machine to machine communication technology to transmit characteristics of products (e.g. manufacturer name, device type or components) and process data (e.g. temperatures, pressures or feed rates). To enable vendor unspecific interoperability the description of product characteristics and process data has to be standardized utilizing technical specifications, the OPC UA companion specifications.

This specification was created by a joint working group of the OPC Foundation and EUROMAP. It is adopted identically as VDMA Specification.

Compared with the previous version, the following changes have been made:

| Version | Changes |
| --- | --- |
| OPC 40082-4, v1.0.1 | Correction in NodeSet: Index of MaterialClass REGRIND_OPEN_LOOP corrected from 4 to 3. Editorial change: Removed chapter headline “Identification and MachineryBuildingBlocks” (former 7.2) Correction of references in the description of 7.1.1 Machine State and Operation Mode |

EUROMAP

EUROMAP is the European umbrella association of the plastics and rubber machinery industry which accounts for annual sales of around 13.5 billion euro and a 40 per cent share of worldwide production. Almost 75 per cent of its European output is shipped to worldwide destinations. With global exports of 10.0 billion euro, EUROMAP’s around 1,000 machinery manufacturers are market leaders with nearly half of all machines sold being supplied by EUROMAP members.

EUROMAP provides technical recommendations for plastics and rubber machines. In addition to standards for machine descriptions, dimensions and energy measurement, interfaces between machines feature prominently. The provision of manufacturer independent interfaces ensures high levels of machine compatibility.

OPC Foundation

OPC is the interoperability standard for the secure and reliable exchange of data and information in the industrial automation space and in other industries. It is platform independent and ensures the seamless flow of information among devices from multiple vendors. The OPC Foundation is responsible for the development and maintenance of this standard.

OPC UA is a platform independent service-oriented architecture that integrates all the functionality of the individual OPC Classic specifications into one extensible framework. This multi-layered approach accomplishes the original design specification goals of:

Platform independence: from an embedded microcontroller to cloud-based infrastructure

Secure: encryption, authentication, authorization and auditing

Extensible: ability to add new features including transports without affecting existing applications

Comprehensive information modelling capabilities: for defining any model from simple to complex

# Scope

OPC 40082-4 describes the data model for dosing systems as part of a plastic & rubber production cell/line (e.g. extrusion/injection moulding) for data exchange with manufacturing execution systems (MES) and other components of the cell/line. Dosed materials may be granulate, powder or liquids. For liquid silicon and rubber dosing systems, please refer to OPC 40082-3: LSR Dosing Systems. This specification is applicable for dosing systems mounted directly on the machine as well as for systems not directly mounted on the machine (e.g. floor-mounted).

The target of OPC 40082-4 is to provide a unique interface for dosing systems from different manufacturers to ensure compatibility.

The following functionalities are covered:

General information about the dosing system (manufacturer, model, serial number…), current configuration and status.

Process information like throughput for monitoring and process optimization.

Closed loop controls between the components of the production cell/line.

Following functions are not included:

Safety related signals like emergency stop

# Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments and errata) applies

OPC 10000-1, OPC Unified Architecture - Part 1: Overview and Concepts

http://www.opcfoundation.org/UA/Part1/

OPC 10000-3, OPC Unified Architecture - Part 3: Address Space Model

http://www.opcfoundation.org/UA/Part3/

OPC 10000-5, OPC Unified Architecture - Part 5: Information Model

http://www.opcfoundation.org/UA/Part5/

OPC 10000-6, OPC Unified Architecture - Part 6: Mappings

http://www.opcfoundation.org/UA/Part6/

OPC 10000-7, OPC Unified Architecture - Part 7: Profiles

http://www.opcfoundation.org/UA/Part7/

OPC 10000-8, OPC Unified Architecture - Part 8: Data Access

http://www.opcfoundation.org/UA/Part8/

OPC 10000-9, OPC Unified Architecture - Part 9: Alarms and Conditions

http://www.opcfoundation.org/UA/Part9/

OPC 10000-100, OPC Unified Architecture - Part 100: Devices

http://www.opcfoundation.org/UA/Part100/

OPC 40001-1, OPC UA for Machinery - Part 1: Basic Building Blocks

http://www.opcfoundation.org/UA/Machinery/

OPC 40083: OPC UA interfaces for plastics and rubber machinery – General Type definitions

http://www.opcfoundation.org/UA/PlasticsRubber/GeneralTypes

# Terms, definitions and conventions

## Overview

It is assumed that basic concepts of OPC UA information modelling are understood in this specification. This specification will use these concepts to describe the OPC 40082-4 Information Model. For the purposes of this document, the terms and definitions given in the documents referenced in Clause 2 apply.

Note that OPC UA terms and terms defined in this specification are italicized in the specification.

## Conventions used in this document

The conventions described in OPC 40083 apply.

## Abbreviations

EXT	Extruder

IMM	Injection Moulding Machine

MES	Manufacturing Execution System

# General information to OPC UA interfaces for plastics and rubber machinery and OPC UA

For general information on OPC UA interfaces for plastics and rubber machinery and OPC UA see OPC 40083.

For liquid silicon and rubber dosing systems, please refer to OPC 40082-3: LSR Dosing Systems.

# Use cases

The following functionalities are covered:

General information about the dosing system (manufacturer, model, serial number… ), current configuration and status.

Optimizing process: Throughput of dosing has to be adjusted with other components in the production cell/line (e.g. extruder/haul-off speed).

Process information (throughput, actual weight)

Monitoring (Status, Errors)

Recipe management: Dosing systems store their configurations in so-called recipes. These include information on nominal process parameters (temperatures, dosing volumes …). OPC 40082-4 allows transferring datasets between dosing systems and MES/line controllers for building a central repository of recipes.

# OPC 40082-4 Information Model overview

The task of a dosing system is to bring material into the production process. This can be done in a continuous or discontinuous manner (batch) or a combination of both (see Figure 1 for an example). The material can be gravity fed or controlled and combined with an optional starve feeder (e.g. for extrusion).

Figure 1 – Dosing System example

The information model defined in this specification models the complex structure of a dosing system as follows:

The top level ObjectType DosingSystemType contains the objects for machine identification, machine state and configuration of the complete dosing system. The individual components are modelled in the OperationType with all properties required to monitor and control the operation of the system.

# DosingSystemType

## DosingSystemType definition

This OPC UA ObjectType is used for the root Object representing a dosing system as part of a production cell/line. It is formally defined in Table 1.

The instance(s) of DosingSystemType shall be located under the Machines Object of the Server (see OPC UA for Machinery).

Figure 2 – Overview DosingSystemType

NOTE: If the OPC UA server is implemented in the controller of the dosing system so only one instance of DosingSystemType will be created. But it is also possible that one OPC UA server is connected to several machine controls as one interface to the MES. In this case several instances of DosingSystemType will be created.

Table 1 – DosingSystemType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | DosingSystemType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of the 0:BaseObjectType defined in OPC 10000-5 |  |  |  |  |  |
| 0:HasAddIn | Object | 2:Identification |  | 3:MachineIdentificationType | M |
| 0:HasComponent | Object | 3:MachineryBuildingBlocks |  | 0:FolderType | M |
| 0:HasComponent | Object | MachineConfiguration |  | 4:MachineConfigurationType | M |
| 0:HasProperty | Variable | DisplayLanguage | 0:LocaleId | 0:PropertyType | O, RW |
| 0:HasProperty | Variable | IsContinuous | 0:Boolean | 0:PropertyType | M, RO |
| 0:HasComponent | Object | Operation |  | OperationType | M |
| 0:HasProperty | Variable | DeviceMappingNumber | 0:UInt32 | 0:PropertyType | M, RW |
| 0:HasComponent | Object | Feeder |  | ContinuousComponentType | O |
| 0:GeneratesEvent | ObjectType | 4:LogbookEventType | Defined in OPC 40083 |  |  |
| 0:GeneratesEvent | ObjectType | 4:MessageConditionType | Defined in OPC 40083 |  |  |
| 0:GeneratesEvent | ObjectType | 4:HelpOffNormalAlarmType | Defined in OPC 40083 |  |  |
| Conformance Units |  |  |  |  |  |
| OPC 40082-4 Basic |  |  |  |  |  |
| OPC 40082-4 Alarms |  |  |  |  |  |

The MachineIdentificationType is defined in OPC UA for Machinery (OPC 40001-1) and provides basic information on a machine/device.

For the InstanceDeclaration the ModellingRules of the Properties Model and DeviceClass are overridden to mandatory.

The Object MachineryBuildingBlocks contains building blocks from OPC UA for Machinery as defined in OPC 40001-1. For this version of OPC 40082-4, the Object uses the two AddIns MachineryItemState and MachineryOperationMode.

Table 2 – DosingSystemType Additional Subcomponents

| BrowsePath | References | NodeClass | BrowseName | DataType | TypeDefinition | Other |
| --- | --- | --- | --- | --- | --- | --- |
| 2:Identification | 0:HasProperty | Variable | 2:Model | 0:LocalizedText | 0:PropertyType | M, RO |
| 2:Identification | 0:HasProperty | Variable | 2:DeviceClass | 0:String | 0:PropertyType | M, RO |
| 3:MachineryBuildingBlocks | 0:HasAddIn | Object | 3:MachineryItemState |  | 3:MachineryItemState_StateMachineType | M |
| 3:MachineryBuildingBlocks | 0:HasAddIn | Object | 3:MachineryOperationMode |  | 3:MachineryOperationModeStateMachineType | M |

The DeviceClass Property shall have the value “Dosing System”.

The components of the DosingSystemType have additional references which are defined in Table 3.

Table 3 – DosingSystemType Additional References

| SourceBrowsePath | Reference Type | Is Forward | TargetBrowsePath |
| --- | --- | --- | --- |
| 3:MachineryBuildingBlocks | 0:HasAddIn | True | 2:Identification |

### Machine State and Operation Mode

To provide the machine state, this specification uses the AddIn MachineryItemState from OPC 40001-1. Different Methods defined in this specification non-exclusively trigger state transitions, meaning that the server can also trigger these transitions in other ways (e.g. HMI input, control logic…):

- EnableDevice (defined in 8.5) triggers the transition from NotAvailable to NotExecuting by e.g. switching on the drives of the device. NotExecuting means, the dosing device is ready for dosing and waiting for a dosing signal.

- DisableDevice (defined in 8.6) triggers the transition from any state to NotAvailable by e.g. switching off the drives of the device. Further SubStates like e.g. “sleep”, “energy saving”, “eco mode” or “off” are not defined in this specification.

- StartDosingShot or StartDosingContinuous (defined in 8.9 and 8.10) trigger the transition from NotExecuting to Executing if RemoteControlOpcUa and EnableDosingOpcUa (defined in 8.7 and 8.8Error! Reference source not found.) are TRUE. If not, the transition is performed by the server.

- The transition from Executing state to any other state can be triggered in different ways depending on the application, e.g. automatically by the server in IMM application after each cycle or by the methods StopDosing or StopDosingAfterCycle in case of continuous dosing.

Figure 3 gives an overview of the machine state, transitions and methods.

Figure 3 – MachineryItemState Overview

For the operation modes, the MachineryOperationMode state machine is used without changes:

Figure 4 – MachineryOperationMode Overview

## DeviceMappingNumber

Description:	Unique identifier/address/number for devices of the same DeviceType within a local network. Several peripheral devices of the same DeviceType can be connected to a production machine (e.g. IMM or extruder). In most applications, the production machine must map the connected peripheral devices to internal logical devices and zones in a fixed configuration (e.g. hot runner systems according to the wiring or temperature control devices according to the tubing).

The mapping shall be stable after reconnecting the devices and is therefore not possible via IP addresses, which can be assigned dynamically via DHCP. DeviceMappingNumber sets the mapping order of peripheral devices of the same type on the local network and is therefore of type UInt32.

Example:	1

## MachineConfiguration

The MachineConfiguration Object represents the current configuration of the Dosing System. The MachineConfigurationType is defined in OPC 40083.

## DisplayLanguage

With the DisplayLanguage Property the client can set the desired language on the user interface at the Dosing System. If the peripheral device does not support the configured language, it can keep the previous setting or use English as the default.

## IsContinuous

The dosing device can be installed on an extruder (EXT) or an injection moulding machine (IMM). EXT is a continuous process, IMM is a cyclic process. For inline dosing devices, directly mounted on the feed throat of an IMM the IsContinuous flag is false, when installed on an EXT this flag is true.

For batch blenders which are cyclic based by themselves, this flag is set true since the output flow of a batch blender does not need to be synchronized with for example an IMM.

## Operation

This ObjectType contains parameters which are necessary to operate the Dosing System. The OperationType is defined in chapter 8.

## Feeder

A feeder can be used for starve feeding (under feeding) of a production machine and is defined by the ContinuousComponentType. E.g. a screw feeder where DosingMode is set to ONLY_CONVEYING. The ContinuousComponentType is defined in chapter11.20.

# OperationType

## OperationType definition

The OperationType contains objects which are necessary to operate the DosingSystem. It is formally defined in Table 4. The top level of the OperationType describes the whole dosing system while detailed information about the single components can be found in the ComponentsType Object.

Figure 5 – OperationType Overview

Table 4 – OperationType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | OperationType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC 10000-5 |  |  |  |  |  |
| 0:HasProperty | Variable | IsActive | 0:Boolean | 0:PropertyType | O, RO |
| 0:HasComponent | Variable | SetActive | 0:Boolean | 0:BaseDataVariableType | O, RW |
| 0:HasComponent | Method | IdentifyDevice |  |  | O |
| 0:HasComponent | Method | EnableDevice |  |  | M |
| 0:HasComponent | Method | DisableDevice |  |  | M |
| 0:HasComponent | Variable | RemoteControlOpcUa | 0:Boolean | 0:BaseDataVariableType | M |
| 0:HasComponent | Variable | EnableDosingOpcUa | 0:Boolean | 0:BaseDataVariableType | M |
| 0:HasComponent | Method | StartDosingShot |  |  | O |
| 0:HasComponent | Method | StartDosingContinuous |  |  | O |
| 0:HasComponent | Method | StopDosing |  |  | O |
| 0:HasComponent | Method | StopDosingAfterCycle |  |  | O |
| 0:HasComponent | Object | Components |  | ComponentsType | M |
| 0:HasComponent | Object | Throughput |  | 4:ControlledParameterType | O |
| 0:HasComponent | Object | ShotWeight |  | 4:ControlledParameterType | O |
| 0:HasComponent | Variable | DosingDuration | 0:Duration | 0:BaseDataVariableType | O, RW |
| 0:HasComponent | Object | Mixing |  | MixingType | O |
| 0:HasComponent | Object | BatchWeight |  | 4:ControlledParameterType | O |
| 0:HasProperty | Variable | HighestActiveAlarmSeverity | 0:UInt16 | 0:PropertyType | M, RO |
| 0:HasComponent | Variable | ActiveErrors | 4:ActiveErrorDataType[] | 0:BaseDataVariableType | M, RO |
| 0:HasComponent | Method | ResetAllErrors |  |  | O |
| 0:HasComponent | Method | ResetErrorById |  |  | O |
| 0:HasComponent | Method | FirstFilling |  |  | O |
| 0:HasComponent | Method | StartPurge |  |  | O |
| 0:HasComponent | Variable | PurgeDuration | 0:Duration | 0:BaseDataVariableType | O |
| 0:HasComponent | Object | ElectricalEnergy |  | 4:EnergyType | O |
| 0:HasComponent | Object | Maintenance |  | DosingMaintenanceType | O |
| 0:HasComponent | Object | ProductionDataSetManagement |  | 4:ProductionDatasetManagementType | O |
| Conformance Units |  |  |  |  |  |
| OPC 40082-4 Basic |  |  |  |  |  |

## IsActive

The IsActive Property provides information if the dosing unit is active in the current production.

## SetActive

The SetActive variable activates the dosing unit for the current production.

NOTE: Malfunction or activation by mistake should be covered on machine side (not part of this specification).

## IdentifyDevice

The DosingSystem on which this method is called shows itself by e.g. activation of a LED.

Signature

IdentifyDevice();

Table 5 – IdentifyDevice Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IdentifyDevice |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

NOTE: This Method is identical to the IdentifyDevice Method in OPC 40082-1.

## EnableDevice

Enables the device, for example activate motor drives. Triggers the transition from MachineryItemState NotAvailable to NotExecuting.

Signature

EnableDevice();

Table 6 – EnableDevice Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | EnableDevice |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## DisableDevice

Disables the device, for example deactivate motor drives. Triggers the transition from any MachineryItemState to NotAvailable.

Signature

DisableDevice();

Table 7 – DisableDevice Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | DisableDevice |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## RemoteControlOpcUa

Variable to set and read the actual remote control state of the server instance. If RemoteControlOpcUa is FALSE, the interface is read-only (Variables not writeable, methods not executable).

RemoteControlOpcUa remains writeable, even if set to FALSE.

## EnableDosingOpcUa

Variable to set and read the actual state how the dosing signal is transferred from the client to the dosing system in addition to RemoteControlOpcUa.

If EnableDosingOpcUa is FALSE, dosing has to be triggered by another interface, e.g. hard-wired signals.

If RemoteControlOpcUa is FALSE, EnableDosingOpcUa shall be ignored.

## StartDosingShot

If EnableDosingOpcUa is TRUE, this method can be used to start dosing for one shot with a defined duration specified by DosingDuration, no arguments.

If EnableDosingOpcUa is FALSE, the return statement shall be “Bad_InvalidState”

In case of inline dosing in a cyclic production process, this method needs to be called every cycle.

Signature

StartDosingShot();

Table 8 – StartDosingShot Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartDosingShot |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

If StartDosingShot is used, DosingDuration becomes mandatory.

## StartDosingContinuous

If EnableDosingOpcUa is TRUE, this method can be used to start dosing continuously, no arguments.

If EnableDosingOpcUa is FALSE, the return statement shall be “Bad_InvalidState”

Signature

StartDosingContinuous();

Table 9 – StartDosingContinuous Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartDosingContinuous |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## StopDosing

Method to stop dosing immediately. No arguments.

Signature

StopDosing();

Table 10 – StopDosing Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StopDosing |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## StopDosingAfterCycle

Method to stop dosing after the current dosing cycle (e.g. to get a valid batch mix with batch blending). No arguments.

Signature

StopDosingAfterCycle();

Table 11 – StopDosingAfterCycle Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StopDosingAfterCycle |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## Components

This ObjectType is a container for all Components in the DosingSystem. The ComponentsType is described in chapter 11.

## Throughput

Current throughput of the DosingSystem in mass or volume per time. The modelling rule for this node is optional to cover pure feeders, but is mandatory for all other dosing units.

Unit: kg/h or lb/h

## ShotWeight

Current shotweight of the parts produced by the production machine in mass.

Unit: g or oz

## DosingDuration

Used for inline dosing where the dosing device is mounted on the feed throat of the production machine. This parameter defines the duration the dosing shall be active after receiving the StartDosingShot signal.

If DosingDuration is used, the dosing device shall stop dosing after the duration. StopDosing or StopDosingAfterCycle are not necessary in this case but shall have a higher priority if used anyway.

## Mixing

In case the dosing station is equipped with an active mixing device like a batch blender, this object provides information about the mixing process. The MixingType is defined in chapter 9.

## BatchWeight

Target batch weight for batch blenders in absolute values.

Unit: g or oz

## HighestActiveAlarmSeverity

Description:	Indication of the severity of the highest active alarm (0 = no active alarm – 1000 = possible error). Together with ActiveErrors, it provides a minimal error handling for devices without alarm support. However, the variable shall be filled even if alarms are supported.

The severity classification defined in OPC 40083 (clause 6.4) shall be used.

Example:	400

## ActiveErrors

Description:	List of the active errors of the device. It provides a minimal error handling for devices without alarm support. However, the variable shall be filled even if alarms are supported. The ActiveErrorDataType is defined in OPC 40083. If there is no active error, the array is empty.

## ResetAllErrors

Description:	Method to reset all errors of the device.

Signature

ResetAllErrors();

## ResetErrorById

Description:	Method to reset one error of the device.

Signature

ResetErrorById(

[in]	String	Id);

Table 12 – ResetErrorById Method Arguments

| Argument | Description |
| --- | --- |
| Id | Id of the error, listed in ActiveErrors, that shall be reset. |

Table 13 – ResetErrorById Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | ResetErrorById |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |
| HasProperty | Variable | InputArguments | Argument[] | PropertyType | Mandatory |

## FirstFilling

Method to fill the DosingSystem for the first time with the mixture of materials from the current recipe.

No Arguments.

Signature

FirstFilling();

Table 14 – FirstFilling Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | FirstFilling |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## StartPurge

Method to start the purging function on the dosing system. No arguments.

Once purging, the State of all components will be set to “Purging” and can be used to monitor if the purging sequence has finished.

Signature

StartPurge();

Table 15 – StartPurge Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartPurge |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## PurgeDuration

Duration of the purging function of the dosing system.

## ElectricalEnergy

Information about the electrical power and energy consumption of the DosingSystem. The EnergyType is defined in OPC 40083.

## Maintenance

Information on the maintenance status of the dosing system. The DosingMaintenanceType is defined in chapter 16.

## ProductionDatasetManagement

The ProductionDatasetManagementType is defined in OPC 40083 and provides functionalities for the management of recipes/machine settings.

# MixingType

## MixingType definition

In case the dosing station is equipped with an active mixing device like on a batch blender, the mixing process can be controlled with the type MixingType, which is defined in Table 16.

Table 16 – MixingType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | MixingType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC UA Part 5 |  |  |  |  |  |
| 0:HasComponent | Variable | MixingMode | 0:UInt16 | 0:MultiStateValueDiscreteType | M, RW |
| 0:HasProperty | Variable | MixingInterval | 0:UInt16 | 0:PropertyType | O, RW |
| 0:HasProperty | Variable | MixingDuration | 0:Duration | 0:PropertyType | O, RW |
| 0:HasComponent | Variable | IdleMixing | 0:Boolean | 0:BaseDataVariableType | O, RW |

## MixingMode

The mixing mode parameter can be used to set the required mixing mode. The MultiStateValueDiscreteType should provide the available mixing modes.

Table 17 – Values for MixingMode

| EnumValue | ValueAsText | Description |
| --- | --- | --- |
| 0 | NO_MIXING | No mixer available or mixing OFF |
| 1 | CONTINUOUS_MIXING | Mixer is activated after the first batch (blender) and is mixing continuous until the dosing device is stopped |
| 2 | TIMED_MIXING | The mixer is started after each batch (blender) and mixes for a defined duration (MixingDuration 9.4) |
| 3 | INTERVAL_MIXING | The mixer device is started after a defined number of shots, IMM (MixingInterval 9.3). Duration of mixing is set by MixingDuration |

## MixingInterval

Interval between two mixing processes, in number of shots

Note: Mandatory if MixingMode is set to INTERVAL_MIXING

## MixingDuration

Duration of a mixing process

Note: Mandatory, if MixingMode is set to TIMED_MIXING or INTERVAL_MIXING

## IdleMixing

This flag can be used for “idle mixing”. With idle mixing the mixer of the batch blender rotates once in a while to keep the material in the mixer flowing and material does not get blocked by the mixer blades.

# ComponentsType

## ComponentsType definition

This ObjectType is a container for all Components in the DosingSystem. It is formally defined in Table 18.

Table 18 – ComponentsType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | ComponentsType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC 10000-5 |  |  |  |  |  |
| 0:HasProperty | Variable | 0:NodeVersion | 0:String | 0:PropertyType | M, RO |
| 0:HasComponent | Object | <Component_Nr> |  | ComponentType | MP |
| 0:GeneratesEvent | ObjectType | 0:GeneralModelChangeEventType | Defined in OPC 10000-5 |  |  |

Conformance Units

OPC 40082-4 Basic

When instances for components are created, the BrowseNames shall be “<Component_Nr>” where Nr is a three-digit number with leading zeros, starting with “001”. The ComponentType is defined in chapter 11.

# ComponentType

## ComponentType definition

The ComponentType represents a device which brings material into the dosing system. For the component name, the Node Attribute DisplayName shall be used.

The ComponentType is abstract. A derived subtype, either for continuous dosing (ContinuousComponentType) or discontinuous dosing (BatchComponentType) shall be used.

Free falling components shall be modelled with DosingMode “Only_Conveying”.

Figure 6 – ComponentType Overview

Table 19 – ComponentType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | ComponentType |  |  |  |  |
| IsAbstract | True |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC 10000-5 |  |  |  |  |  |
| 0:HasProperty | Variable | IsPresent | 0:Boolean | 0:PropertyType | M, RO |
| 0:HasProperty | Variable | IsActive | 0:Boolean | 0:PropertyType | O, RO |
| 0:HasComponent | Variable | SetActive | 0:Boolean | 0:BaseDataVariableType | O, RW |
| 0:HasComponent | Variable | State | ComponentStateEnumeration | 0:BaseDataVariableType | M, RO |
| 0:HasProperty | Variable | FeedRateScalingFactor | 0:Double | 0:PropertyType | O, RW |
| 0:HasComponent | Object | Throughput |  | 4:ControlledParameterType | O |
| 0:HasComponent | Variable | DosingMode | 0:UInt16 | 0:MultiStateValueDiscreteType | M, RW |
| 0:HasComponent | Object | ElectricalEnergy |  | 4:EnergyType | O |
| 0:HasComponent | Object | Material |  | DosingMaterialType | O |
| 0:HasComponent | Object | MaterialConsumption |  | MaterialConsumptionType | O |
| 0:HasComponent | Variable | Weight | 0:Double | 0:AnalogUnitType | O, RO |
| 0:HasComponent | Variable | BulkDensity | 0:Double | 0:AnalogUnitType | O, RW |
| 0:HasComponent | Method | IdentifyComponent |  |  | O |
| 0:HasComponent | Method | StartCleaning |  |  | O |
| 0:HasComponent | Method | AbortCleaning |  |  | O |
| 0:HasComponent | Object | Levels |  | LevelsType | O |
| 0:HasComponent | Method | StartCalibration |  |  | O |
| 0:HasComponent | Method | AbortCalibration |  |  | O |
| 0:HasSubtype | ObjectType | ContinuousComponentType | Defined in 11.20 |  |  |
| 0:HasSubtype | ObjectType | BatchComponentType | Defined in 11.21 |  |  |
| Conformance Units |  |  |  |  |  |
| OPC 40082-4 Basic |  |  |  |  |  |

## IsPresent

The IsPresent flag indicates if the hardware of this component is installed. This IsPresent system is used for those Dosing equipment manufacturers using a fixed OPC UA model for different hardware models

## SetActive

The SetActive variable activates the dosing component for the current production. SetActive = False deactivates the component, even if the setpoint is bigger than 0 % in the recipe.

NOTE: In some cases where the dosing Percentage setpoint of this component is bigger than 0 % and SetActive = False, the real total material output of the dosing station is not 100 % but 100 %-x % of this component.

Example: a dosing station with 3 components, respectively 80%, 15%, 5% and an Extruder Throughput demand of 100 kg/h. The component of 5 % SetActive = False. The dosing system will actually supply 95 % (95 kg/h).

## IsActive

The IsActive Property provides information if the dosing component is active in the current production.

## State

The State property of this component to read the actual working state of the component.

Table 20 – ComponentStateEnumeration Definition

| Name | Value | Description |
| --- | --- | --- |
| Off | 0 | The component has been switched off |
| Ready | 1 | The component is waiting for a start dosing signal |
| Dosing | 2 | The component is actively feeding material |
| Calibration | 3 | Component is in calibration mode and can typically not be used for dosing or filling |
| Error | 4 | The component has an active error |
| Cleaning | 5 | The component is in cleaning mode e.g. hopper draining |
| Purging | 6 | The component is in purging mode |
| Priming | 7 | The component is in priming mode |

## FeedrateScalingFactor

Material-specific output of the Component per DrivePercentage.

Example:

A dosing screw with a maximum rotational speed of 300 rpm, the material output flow is 900 grams per second.  300 rpm means 100 % DrivePercentage. The Feedrate would then be 900/100 = 9 gram per % per second (g/(%*s))

Running at 50 % DrivePercentage, the output will be 50*9 = 450 g/s.

Example:

A vibrational feeder running at 50 Hz, the material output flow is 10 gram per second. The maximum frequency of the feeder is 250 Hz. 250 Hz equals 100 % DrivePercentage, 50 Hz equals 20 % DrivePercentage. The feederate is 10/20 = 0.5 gram per % per second (g/%/s).

## Throughput

Material Output per time.

Unit: kg/h or lb/h

## DosingMode

The DosingMode Property provides information, how the dosing of the component is controlled.

Table 21 – DosingModeEnumeration Definition

| Name | Value | Description |
| --- | --- | --- |
| ONLY_CONVEYING | 0 | The throughput is not controlled. The feeder only transports the material (e.g. by screw, conveyor belt) or the material is only falling through a feed opening. |
| VOLUMETRIC | 1 | The throughput is controlled by a volumetric dosing system. |
| GRAVIMETRIC | 2 | The throughput is controlled by a gravimetric dosing system. |
| OTHER | 3 | Throughput is controlled, but in another mode than these above. |

## ElectricalEnergy

Information about the electrical power and energy consumption of the component. The EnergyType is defined in OPC 40083.

## Material

Information about the material used in the component. The MaterialType is defined in OPC 40083.

## Weight

Weight of the Material (e.g. inside the hopper).

Unit: g or oz

## BulkDensity

Bulk density of the material which is dosed (not to be confused with the density of the material itself defined in MaterialType).

Unit: kg/l or lb/gal or lb/ft³

## IdentifyComponent

The dosing component on which this method is called shows itself by e.g. activation of a LED.

Signature

IdentifyComponent();

NOTE: This Method is identical to the IdentifyDevice Method in OPC 40082-1.

Table 22 – IdentifyComponent Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | IdentifyComponent |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## StartCleaning

Start the automated cleaning sequence for this component. Once cleaning the component State will be set to Cleaning and can be used to monitor if cleaning sequence has finished.

Method without arguments.

Signature

StartCleaning();

Table 23 – StartCleaning Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartCleaning |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## AbortCleaning

Method without arguments to abort the current cleaning sequence. If the device MachineryItemState is NotAvailable, the component State shall switch to Off, otherwise to Ready.

Signature

AbortCleaning();

Table 24 – AbortCleaning Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | AbortCleaning |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## Levels

Information about the filling of the dosing component. The LevelsType is defined in chapter 12.

## MaterialConsumption

Information about the material consumption of the component. The MaterialConsumptionType is defined in chapter 13.

## StartCalibration

Start the automated calibration sequence for this component. Once calibrating the State will be set to “Calibration” and can be used to monitor if the calibration sequence has finished.

Material and BulkDensity should be supported.

Method without arguments.

Signature

StartCalibration();

Table 25 – StartCalibration Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartCalibration |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## AbortCalibration

Abort the current calibration sequence for this component.

Method without arguments.

Signature

AbortCalibration();

Table 26 – AbortCalibration Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartCalibration |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## ContinuousComponentType

### ContinuousComponentType definition

The ContinuousComponentType is a subtype of the abstract ComponentType and describes Components, where the materials are fed to the production machine in a continuous flow instead of batches. Injection moulding can be considered as continuous flow during plastification.

The ContinuousComponentType is formally defined in 11.20.

The dosing can be controlled with either (and only one of) Percentage or DrivePercentage.

Table 27 – ContinuousComponentType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | ContinuousComponentType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of ComponentType defined in 11 |  |  |  |  |  |
| 0:HasComponent | Object | Percentage |  | 4:ControlledParameterType | O |
| 0:HasComponent | Object | WeighingProcess |  | WeighingProcessType | O |
| 0:HasComponent | Object | DrivePercentage |  | 4:ControlledParameterType | O |
| 0:HasComponent | Method | StartPriming |  |  | O |
| 0:HasComponent | Method | StopPriming |  |  | O |

### Percentage

Part (%) of the recipe dosed by this component (recipe).

### WeighingProcess

The weighing process can be used to perform a material pre-calibration or to determine the feedrate of the dosing system used by this component.

The WeighingProcessType is defined in chapter 15.

### DrivePercentage

Relative value (0-100%) of the maximum drive power of the component drive.

Example: A dosing screw with a maximum rotational speed of 300 rpm. 300 rpm equals 100% DrivePercentage. At 50% DrivePercentage the screw rotates 150 rpm.

### StartPriming

Method to start the priming function on the component. No arguments.

Once priming, the component State will be set to “Priming” and can be used to monitor if priming is active.

Signature

StartPriming();

Table 28 – StartPrimingMethod AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartPriming |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

### StopPriming

Method to stop the priming function on the component. No arguments.

Signature

StopPriming();

Table 29 – StopPrimingMethod AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StopPriming |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## BatchComponentType

### BatchComponentType definition

The BatchComponentType is a subtype of the abstract ComponentType and describes Components, where the materials are fed to the production machine in batches e.g. prepared by a batch blender.

Table 30 – BatchComponentType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | BatchComponentType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of ComponentType defined in 11 |  |  |  |  |  |
| 0:HasComponent | Object | Percentage |  | 4:ControlledParameterType | M |

### Percentage

Part (%) of the recipe dosed by this component (recipe).

# LevelsType

## LevelsType definition

The LevelsType gives information about the filling of the dosing component and is described in Table 31.

Table 31 – LevelsType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | LevelsType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC UA Part 5 |  |  |  |  |  |
| 0:HasComponent | Object | Level |  | 4:MonitoredParameterType | M |
| 0:HasComponent | Variable | Filling | 0:Boolean | 0:BaseDataVariableType | O |
| 0:HasComponent | Method | StartFilling |  |  | O |
| 0:HasComponent | Method | AbortFilling |  |  | O |

## Level

The actual level of the material inside the component (e.g. actual hopper level).

Values are given as relative values between 0 – 100 %.

MinValue of MonitoredParameterType is used to trigger an alarm.

## Filling

Flag to indicate if the hopper is being filled right now.

## StartFilling

Method to fill the Component up to the MaxValue of the MonitoredParameterType in the Level Variable.

No Arguments.

Signature

StartFilling();

Table 32 – StartFilling Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | StartFilling |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

## AbortFilling

Method to abort filling the Component.

No Arguments.

Signature

AbortFilling();

Table 33 – AbortFilling Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | AbortFilling |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Modelling Rule |

# DosingMaterialType

## DosingMaterialType definition

Provides information about the material. It is a Subtype of the MaterialType defined in OPC 40083.

Table 34 – DosingMaterialType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | DosingMaterialType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 4:MaterialType defined in OPC 40083 |  |  |  |  |  |
| 0:HasComponent | Variable | MaterialClass | 0:UInt16 | 0:MultiStateValueDiscreteType | M |

## MaterialClass

Provides information about the material class, which can be used for the recipe calculation by the server or client. The TypeDefinition is 0:MultiStateValueDiscreteType, values are given in Table 35. Vendor specific classes can be used with EnumValue ≥ 100.

Table 35 – Values for MaterialClass

| EnumValue | ValueAsText | Description |
| --- | --- | --- |
| 0 | MAIN | Main material |
| 1 | ADDITIVE | Additive e.g. coloring material |
| 2 | REGRIND_CLOSED_LOOP | Regrind with known composition, which is already considered in the internal dosing control and therefore excluded from the recipe calculation. |
| 3 | REGRIND_OPEN_LOOP | Regrind with unknown composition. Included in the recipe calculation. |

EXAMPLE:

With a recipe consisting of additive A = 9%, additive B = 10% and regrind ratio = 25%, the Percentage values calculate depending on the MaterialClass of the regrind material, shown in Table 36.

If the regrind composition is known (REGRIND_CLOSED_LOOP), e.g. sprue regrind in a stable process, where the regrind has the same composition as the recipe, only the main material share has to be mixed with the additives. The internal calculation percentage of Additive A calculates to 9 % × 75 % = 6,75 %, since 25 % of the additive (2,25 % in total) come from the regrind.

If the regrind composition is unknown (REGRIND_OPEN_LOOP), the regrind is treated as an additional component.

Table 36: Calculation Example based on Regrind MaterialClass

| Component | MaterialClass | Recipe | MaterialClass of Regrind |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | REGRIND_CLOSED_LOOP |  | REGRIND_OPEN_LOOP |  |
|  |  |  | Percentage | Internal calculation | Percentage | Internal calculation |
| Main | MAIN | - | 81 % | 60,75 % | 56 % | 56 % |
| Additive A | ADDITIVE | 9 % | 9 % | 6,75 % | 9 % | 9 % |
| Additive B | ADDITIVE | 10 % | 10 % | 7,5 % | 10 % | 10 % |
| Regrind |  | 25 % | 25 % | 25 % | 25 % | 25 % |
| ∑ |  |  | 125 % |  | 100 % |  |

# MaterialConsumptionType

## MaterialConsumptionType definition

Provides information about the material consumption.

Table 37 – MaterialConsumptionType definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | MaterialConsumptionType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC UA Part 5 |  |  |  |  |  |
| 0:HasComponent | Variable | TotalConsumption | 0:Double | 0:AnalogUnitType | M, RO |
| 0:HasComponent | Variable | Scrap | 0:Double | 0:AnalogUnitType | O, RO |
| 0:HasComponent | Method | Reset |  |  | M |

## TotalConsumption

Total mass or volume of fed material (good + scrap) since last reset.

Units for mass: kg or lb

Units for volume: cm³ or in³

## Scrap

Total mass or volume of fed scrap since last reset.

Same units as TotalConsumption.

## Reset

The Method Reset resets the two counters TotalConsumption and Scrap. The signature of this Method is specified below. Table 38 specifies the AdressSpace representation.

Signature

Reset()

Table 38 – Reset Method AddressSpace Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | Reset |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | ModellingRule |

# WeighingProcessType

## WeighingProcessType definition

The weighing process can be used to perform a material pre-calibration or to determine the feedrate of the dosing system used by this component.

Table 39 – WeighingProcessType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | WeighingProcessType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 0:BaseObjectType defined in OPC 10000-5 |  |  |  |  |  |
| 0:HasComponent | Variable | TestWeight | 0:Double | 0:AnalogUnitRangeType | M, RW |
| 0:HasProperty | Variable | WeighingTime | 0:Duration | 0:PropertyType | M, RW |

## TestWeight

Weight of the material of this component.

Unit: g or oz

## WeighingTime

Duration of the weighing process.

# DosingMaintenanceType

## DosingMaintenanceType definition

The DosingMaintenanceType is a Subtype of the MaintenanceType, which is defined in OPC 40083. The Inverval Variable is mandatory and read/writeable in this subtype.

Table 40 – DosingMaintenanceType Definition

| Attribute | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BrowseName | DosingMaintenanceType |  |  |  |  |
| IsAbstract | False |  |  |  |  |
| References | Node Class | BrowseName | DataType | TypeDefinition | Other |
| Subtype of 4:MaintenanceType defined in 40083 |  |  |  |  |  |
| 0:HasComponent | Variable | 4:Interval | 0:Double | 0:AnalogItemType | M, RW |

# Alarmmanagement

All alarms shall be from the HelpOffNormalAlarmType defined in OPC 40083.

As defined in OPC 40083, the root node of the specific interface, e.g. an instance of DosingSystemType, sets the SubscribeToEvents flag in the EventNotifier attribute.

The client subscribes to events at this root node and receives the events already defined in this specification, such as temperature limit alarms or diagnostic events.

In the case of Component related messages, the SourceNode (included in BaseEventType) shall contain the NodeId of the related component. In case of medium or high severity, the production machine can sort out bad parts or stop production.

# Profiles and Conformance Units

This chapter defines the corresponding profiles and conformance units for the OPC UA Information Model for OPC 40082-4. Profiles are named groupings of conformance units. Facets are profiles that will be combined with other Profiles to define the complete functionality of an OPC UA Server or Client.

## Conformance Units

This chapter defines the corresponding Conformance Unit for OPC 40082-4.

Table 41 – Conformance Units for OPC 40082-4

| Category | Title | Description |
| --- | --- | --- |
| Server | OPC 40082-4 Basic | Support of DosingSystemType and all mandatory child elements giving information on the dosing system and its status. There is at least one instance of the DosingSystemType in the Machines Object. |
| Server | OPC 40082-4 Alarms | Support of HelpOffNormalAlarmType, providing error information. If this facet is supported and a client subscribes to the events, the server shall provide all errors via alarms in addition to the error variables included in the OperationType. |

## Profiles

### Profile list

The following tables specify the facets available for Servers that implement the OPC 40082-4 Information Model companion specification.

NOTE: The names of the supported profiles are available in the Server Object under ServerCapabilities.ServerProfileArray

Table 42 lists all Profiles defined in this document and defines their URIs.

Table 42 – Profile URIs for OPC 40082-4

| Profile | URI |
| --- | --- |
| OPC 40082-4 Basic Server Profile | http://opcfoundation.org/UA-Profile/PlasticsRubber/Dosing/Server/Basic |
| OPC 40082-4 Alarms Server Facet | http://opcfoundation.org/UA-Profile/PlasticsRubber/Dosing/Server/Alarms |

### Server Facets

#### Overview

The following sections specify the Facets available for Servers that implement the OPC 40082-4 companion specification. Each section defines and describes a Facet or Profile.

#### OPC 40082-4 Basic Server Profile

Table 43 – OPC 40082-4 Basic Server Profile

| Group | Conformance Unit / Profile Title | Mandatory / Optional |
| --- | --- | --- |
| OPC 40082-4 | OPC 40082-4 Basic | M |
| Profile | 0:ComplexType Server Facet | M |
| Profile | 0:Method Server Facet | M |
| Profile | 2:BaseDevice Server Facet | M |

#### OPC 40082-3 Alarms Server Facet

Table 44 – OPC 40082-4 Alarms Server Facet

| Group | Conformance Unit / Profile Title | Mandatory / Optional |
| --- | --- | --- |
| OPC 40082-4 | OPC 40082-4 Alarms | M |
| Profile | 0:A & C Alarm Server Facet | M |

# Namespaces

## Namespace Metadata

Table 45 defines the namespace metadata for this document. The Object is used to provide version information for the namespace and an indication about static Nodes. Static Nodes are identical for all Attributes in all Servers, including the Value Attribute. See OPC 10000-5 for more details.

The information is provided as Object of type NamespaceMetadataType. This Object is a component of the Namespaces Object that is part of the Server Object. The NamespaceMetadataType ObjectType and its Properties are defined in OPC 10000-5.

The version information is also provided as part of the ModelTableEntry in the UANodeSet XML file. The UANodeSet XML schema is defined in OPC 10000-6.

Table 45 – NamespaceMetadata Object for this Document

| Attribute | Value |  |  |
| --- | --- | --- | --- |
| BrowseName | http://opcfoundation.org/UA/PlasticsRubber/Dosing/ |  |  |
| Property |  | DataType | Value |
| NamespaceUri |  | String | http://opcfoundation.org/UA/PlasticsRubber/Dosing/ |
| NamespaceVersion |  | String | 1.0.1 |
| NamespacePublicationDate |  | DateTime | 2026-10-01 |
| IsNamespaceSubset |  | Boolean | False |
| StaticNodeIdTypes |  | IdType [] | 0 |
| StaticNumericNodeIdRange |  | NumericRange [] |  |
| StaticStringNodeIdPattern |  | String |  |

Note: The IsNamespaceSubset Property is set to False as the UANodeSet XML file contains the complete Namespace. Servers only exposing a subset of the Namespace need to change the value to True.

## Handling of OPC UA Namespaces

Namespaces are used by OPC UA to create unique identifiers across different naming authorities. The Attributes NodeId and BrowseName are identifiers. A Node in the UA AddressSpace is unambiguously identified using a NodeId. Unlike NodeIds, the BrowseName cannot be used to unambiguously identify a Node. Different Nodes may have the same BrowseName. They are used to build a browse path between two Nodes or to define a standard Property.

Servers may often choose to use the same namespace for the NodeId and the BrowseName. However, if they want to provide a standard Property, its BrowseName shall have the namespace of the standards body although the namespace of the NodeId reflects something else, for example the EngineeringUnits Property. All NodeIds of Nodes not defined in this document shall not use the standard namespaces.

Table 46 provides a list of mandatory and optional namespaces used in an OPC 40082-4 OPC UA Server.

Table 46 – Namespaces used in a OPC 40082-4 Server

| NamespaceURI | Description | Use |
| --- | --- | --- |
| http://opcfoundation.org/UA/ | Namespace for NodeIds and BrowseNames defined in the OPC UA specification. This namespace shall have namespace index 0. | Mandatory |
| Local Server URI | Namespace for nodes defined in the local server. This namespace shall have namespace index 1. | Mandatory |
| http://opcfoundation.org/UA/DI/ | Namespace for NodeIds and BrowseNames defined in OPC 10000-100. The namespace index is Server specific. | Mandatory |
| http://opcfoundation.org/UA/Machinery/ | Namespace for NodeIds and BrowseNames defined in OPC UA for Machinery – Part 1: Basic Building Blocks (OPC 40001-1). The namespace index is Server specific. | Mandatory |
| http://opcfoundation.org/UA/PlasticsRubber/ GeneralTypes/ | Namespace for NodeIds and BrowseNames defined in OPC 40083. The namespace index is server specific. | Mandatory |
| http://opcfoundation.org/UA/PlasticsRubber/Dosing/ | Namespace for NodeIds and BrowseNames defined in this document. The namespace index is Server specific. | Mandatory |
| Vendor specific types | A Server may provide vendor-specific types like types derived from ObjectTypes defined in this document in a vendor-specific namespace. | Optional |
| Vendor specific instances | A Server provides vendor-specific instances of the standard types or vendor-specific instances of vendor-specific types in a vendor-specific namespace. It is recommended to separate vendor specific types and vendor specific instances into two or more namespaces. | Mandatory |

Table 47 provides a list of namespaces and their indices used for BrowseNames in this document. The default namespace of this document is not listed since all BrowseNames without prefix use this default namespace.

Table 47 – Namespaces used in this document

| NamespaceURI | Namespace Index | Example |
| --- | --- | --- |
| http://opcfoundation.org/UA/ | 0 | 0:EngineeringUnits |
| http://opcfoundation.org/UA/DI/ | 2 | 2:DeviceClass |
| http://opcfoundation.org/UA/Machinery/ | 3 | 3:MachineIdentificationType |
| http://opcfoundation.org/UA/PlasticsRubber/GeneralTypes/ | 4 | 4:MachineInformationType |

(normative)

OPC 40082-4 Namespace and mappings

NodeSet and supplementary files for OPC 40082-4 Information Model

The OPC 40082-4 Information Model is identified by the following URI:

http://opcfoundation.org/UA/PlasticsRubber/Dosing/

Documentation for the NamespaceUri can be found here.

The NodeSet associated with this version of specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/PlasticsRubber/Dosing/&v=1.0.1&i=1

The NodeSet associated with the latest version of the specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/PlasticsRubber/Dosing/&i=1

Supplementary files for the OPC 40082-4 Information Model can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/PlasticsRubber/Dosing/&v=1.0.1&i=2

The files associated with the latest version of the specification can be found here:

https://reference.opcfoundation.org/nodesets/?u=http://opcfoundation.org/UA/PlasticsRubber/Dosing/&i=2

___________